Communication Protocols

Complete Reference Manual

Architecture · Frame Structures · Block Diagrams · C Code Examples

30+ Protocols across 7 Domains

Version 1.0 — September 2026

# Table of Contents

Table of Contents	2

Protocol Index	6

Module 1: Automotive Protocols	7

1.1 CAN	7

Overview	7

Block Diagram	7

Frame Structure (Bit-Level)	7

C Code Example	7

Viva Question	8

1.2 LIN	8

Overview	8

Block Diagram	8

Frame Structure (Bit-Level)	8

C Code Example	9

Viva Question	9

1.3 FlexRay	9

Overview	9

Block Diagram	9

Frame Structure (Bit-Level)	9

C Code Example	10

Viva Question	10

1.4 MOST	10

Overview	10

Frame Structure (Bit-Level)	10

Viva Question	10

1.5 Automotive Ethernet	10

Overview	10

Frame Structure (Bit-Level)	11

Viva Question	11

Module 2: Industrial Automation Protocols	12

2.1 Modbus	12

Overview	12

Frame Structure (Bit-Level)	12

C Code Example	12

Viva Question	12

2.2 PROFINET	12

Overview	13

Frame Structure (Bit-Level)	13

Viva Question	13

2.3 EtherCAT	13

Overview	13

Frame Structure (Bit-Level)	13

Viva Question	13

2.4 EtherNet/IP	14

Overview	14

Viva Question	14

2.5 BACnet	14

Overview	14

Viva Question	14

Module 3: IoT & Wireless Protocols	15

3.1 MQTT	15

Overview	15

Frame Structure (Bit-Level)	15

C Code Example	15

Viva Question	15

3.2 CoAP	15

Overview	16

Frame Structure (Bit-Level)	16

Viva Question	16

3.3 Zigbee	16

Overview	16

Viva Question	16

3.4 Z-Wave	16

Overview	16

Frame Structure (Bit-Level)	17

Viva Question	17

3.5 LoRaWAN	17

Overview	17

Frame Structure (Bit-Level)	17

Viva Question	17

3.6 BLE	18

Overview	18

Frame Structure (Bit-Level)	18

Viva Question	18

Module 4: Web & Application Protocols	19

4.1 HTTP/HTTPS	19

Overview	19

Frame Structure (Bit-Level)	19

Viva Question	19

4.2 WebSocket	19

Overview	19

Frame Structure (Bit-Level)	19

Viva Question	19

4.3 gRPC	20

Overview	20

Viva Question	20

4.4 GraphQL	20

Overview	20

Viva Question	20

Module 5: IT, Networking & Infrastructure	21

5.1 TCP/IP	21

Overview	21

Block Diagram	21

Frame Structure (Bit-Level)	21

Viva Question	21

5.2 DNS	21

Overview	21

Frame Structure (Bit-Level)	21

Viva Question	22

5.3 DHCP	22

Overview	22

Viva Question	22

5.4 BGP	22

Overview	22

Frame Structure (Bit-Level)	22

Viva Question	22

5.5 SNMP	23

Overview	23

Viva Question	23

Module 6: File Transfer & Storage Protocols	24

6.1 FTP / SFTP	24

Overview	24

Viva Question	24

6.2 NFS	24

Overview	24

Viva Question	24

6.3 SMB	24

Overview	24

Frame Structure (Bit-Level)	24

Viva Question	25

6.4 iSCSI	25

Overview	25

Frame Structure (Bit-Level)	25

Viva Question	25

Module 7: Communication & Messaging Protocols	26

7.1 SMTP / IMAP / POP3	26

Overview	26

Viva Question	26

7.2 SIP	26

Overview	26

Viva Question	26

7.3 XMPP	26

Overview	26

Viva Question	26

Appendix: Protocol Speed Quick-Reference	27

# Protocol Index

| # | Module | Protocols |
| --- | --- | --- |
| 1 | Automotive | CAN, LIN, FlexRay, MOST, Automotive Ethernet |
| 2 | Industrial Automation | Modbus, PROFINET, EtherCAT, EtherNet/IP, BACnet |
| 3 | IoT & Wireless | MQTT, CoAP, Zigbee, Z-Wave, LoRaWAN, BLE |
| 4 | Web & Application | HTTP/HTTPS, WebSocket, gRPC, GraphQL |
| 5 | Networking & Infrastructure | TCP/IP, DNS, DHCP, BGP, SNMP |
| 6 | File Transfer & Storage | FTP/SFTP, NFS, SMB, iSCSI |
| 7 | Communication & Messaging | SMTP/IMAP/POP3, SIP, XMPP |

# Module 1: Automotive Protocols

## 1.1 CAN

Controller Area Network

### Overview

OSI Layer: Layer 1 (Physical) & Layer 2 (Data Link)

Topology: Multi-master linear bus

Medium: Twisted pair (CAN_H, CAN_L) with 120Ω termination

Arbitration: CSMA/CR — bitwise arbitration, lowest ID wins

Speed / Baud Rate: Up to 1 Mbps (Classic CAN), 8 Mbps data phase (CAN FD)

### Block Diagram

┌──────────┐    ┌──────────┐    ┌──────────┐

│  ECU #1  │    │  ECU #2  │    │  ECU #3  │

│(Engine)  │    │ (Brakes) │    │  (ABS)   │

└────┬─────┘    └────┬─────┘    └────┬─────┘

│               │               │

CAN_H ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━ [120Ω]

CAN_L ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━ [120Ω]

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| SOF | 1 | Dominant bit (0) — start of frame |
| Identifier | 11 | Priority/ID (lower = higher priority) |
| RTR | 1 | 0=Data, 1=Remote |
| IDE | 1 | 0=Standard (11-bit), 1=Extended (29-bit) |
| r0 | 1 | Reserved (dominant) |
| DLC | 4 | Data Length Code (0–8 bytes) |
| Data Field | 0–64 | Payload: 0 to 8 bytes |
| CRC Sequence | 15 | Cyclic Redundancy Check |
| CRC Delimiter | 1 | Recessive bit |
| ACK Slot | 1 | Receivers assert dominant |
| ACK Delimiter | 1 | Recessive bit |
| EOF | 7 | 7 recessive bits |
| Total (max) | 108 | Standard frame with 8-byte data |

### C Code Example

/* CAN Transmission — SocketCAN (Linux) */

#include <stdio.h>

#include <string.h>

#include <linux/can.h>

#include <linux/can/raw.h>

#include <sys/socket.h>

#include <net/if.h>

#include <sys/ioctl.h>

#include <unistd.h>

int can_send(const char *iface, uint32_t id,

uint8_t *data, uint8_t len) {

int sock = socket(PF_CAN, SOCK_RAW, CAN_RAW);

struct ifreq ifr;

struct sockaddr_can addr;

struct can_frame frame;

strncpy(ifr.ifr_name, iface, IFNAMSIZ - 1);

ioctl(sock, SIOCGIFINDEX, &ifr);

addr.can_family  = AF_CAN;

addr.can_ifindex = ifr.ifr_ifindex;

bind(sock, (struct sockaddr *)&addr, sizeof(addr));

frame.can_id  = id;       /* 11-bit standard ID */

frame.can_dlc = len;

memcpy(frame.data, data, len);

write(sock, &frame, sizeof(frame));

close(sock);

return 0;

}

int main(void) {

uint8_t payload[] = {0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08};

can_send("can0", 0x123, payload, 8);

return 0;

}

### Viva Question

Q: "How does CAN handle collisions?"

A: CAN uses bitwise arbitration via the Identifier field. Dominant bits (0) overwrite recessive bits (1), so the message with the lowest ID value wins the bus without data corruption.

## 1.2 LIN

Local Interconnect Network

### Overview

OSI Layer: Layer 1 & 2

Topology: Single-master, multi-slave

Medium: Single wire + ground

Signaling: NRZ, dominant=0V, recessive=V_BAT (~12V)

Speed / Baud Rate: Max 20 kbps

### Block Diagram

┌────────────────┐

│   LIN Master   │

└──────┬─────────┘

│  LIN Bus (Single Wire)

┌────┴────┬──────────┬──────────┐

┌─┴───┐  ┌─┴───┐  ┌───┴──┐  ┌───┴──┐

│Mirr.│  │Wind.│  │Wiper │  │ Seat │

└─────┘  └─────┘  └──────┘  └──────┘

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| Break Field | ≥13 | Dominant bits signaling frame start |
| Sync Field | 8 | Fixed 0x55 for baud rate detection |
| PID | 8 | 6-bit ID + 2 parity bits |
| Data | 8–64 | 1 to 8 bytes (slave response) |
| Checksum | 8 | Classic or Enhanced |

### C Code Example

/* LIN PID Calculation & Checksum */

#include <stdint.h>

uint8_t lin_calc_pid(uint8_t id) {

id &= 0x3F;

uint8_t p0 = ((id>>0)^(id>>1)^(id>>2)^(id>>4)) & 1;

uint8_t p1 = ~((id>>1)^(id>>3)^(id>>4)^(id>>5)) & 1;

return id | (p0 << 6) | (p1 << 7);

}

uint8_t lin_checksum(uint8_t pid,

const uint8_t *data, uint8_t len) {

uint16_t sum = pid;

for (uint8_t i = 0; i < len; i++) {

sum += data[i];

if (sum > 0xFF) sum -= 0xFF;

}

return (uint8_t)(~sum);

}

### Viva Question

Q: "Why use LIN when you already have CAN?"

A: Cost reduction. LIN eliminates expensive crystal oscillators and uses a single wire, perfect for non-safety subsystems like power windows.

## 1.3 FlexRay

### Overview

OSI Layer: Layer 1 & 2

Topology: Dual-channel bus/star/hybrid

Medium: 100Ω differential twisted pair per channel

Access: TDMA (static) + FTDMA (dynamic)

Speed / Baud Rate: Up to 10 Mbps/channel (20 Mbps dual)

### Block Diagram

Channel A ═══════════════════════════

│       │       │       │

[Steer] [Brake] [Engine] [Susp.]

│       │       │       │

Channel B ═══════════════════════════

(Redundant Channel)

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| Frame ID | 11 | Slot number (1–2047) |
| Payload Length | 7 | 0–127 words (0–254 bytes) |
| Header CRC | 11 | CRC over header fields |
| Cycle Count | 6 | Current cycle (0–63) |
| Data | 0–2032 | Up to 254 bytes payload |
| Trailer CRC | 24 | CRC over header + data |

### C Code Example

/* FlexRay Header CRC-11 */

uint16_t flexray_hcrc(uint16_t fid,

uint8_t plen, uint8_t sync, uint8_t start) {

uint32_t d = ((uint32_t)sync<<19) |

((uint32_t)start<<18) |

((uint32_t)fid<<7) | plen;

uint16_t crc = 0x01A;

for (int i = 19; i >= 0; i--) {

uint16_t bit = (d>>i)&1;

uint16_t msb = (crc>>10)&1;

crc = (crc<<1)|bit;

if (msb) crc ^= 0x385;

crc &= 0x7FF;

}

return crc;

}

### Viva Question

Q: "What makes FlexRay suitable for drive-by-wire?"

A: Fault tolerance and determinism. Dual-channel redundancy + strict TDMA schedule guarantees critical messages arrive exactly on time.

## 1.4 MOST

Media Oriented Systems Transport

### Overview

OSI Layer: Layers 1–7

Topology: Physical ring

Medium: Polymer Optical Fiber (POF) 650nm

Speed / Baud Rate: MOST25: 25 Mbps, MOST50: 50 Mbps, MOST150: 150 Mbps

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| Preamble | 8+ | Sync pattern |
| Boundary Descriptor | 4 | Sync/async boundary |
| Synchronous Data | 0–384 | Audio channels |
| Asynchronous Data | 0–384 | TCP/IP packets |
| Control Data | 32 | Address + data |
| Total/block | 512 | At 44.1/48 kHz |

### Viva Question

Q: "What happens if one node fails in MOST?"

A: A single node failure breaks the optical ring loop and disrupts the entire network unless a bypass switch is implemented.

## 1.5 Automotive Ethernet

### Overview

Standard: 100BASE-T1 / 1000BASE-T1

Medium: Single UTP, full-duplex

Modulation: PAM-3

Key Feature: TSN (Time-Sensitive Networking)

Speed / Baud Rate: 100 Mbps / 1 Gbps / 10 Gbps

### Frame Structure (Bit-Level)

| Field | Bytes | Description |
| --- | --- | --- |
| Preamble | 7 | 10101010 pattern |
| SFD | 1 | 10101011 |
| Dest MAC | 6 | 48-bit destination |
| Src MAC | 6 | 48-bit source |
| VLAN Tag | 4 | 802.1Q (optional) |
| EtherType | 2 | Protocol ID |
| Payload | 46–1500 | Data |
| FCS | 4 | CRC-32 |
| Total (max) | 1522 | With VLAN |

### Viva Question

Q: "Why replace older buses with Automotive Ethernet?"

A: Bandwidth. ADAS sensors generate gigabits/s that legacy CAN cannot handle.

# Module 2: Industrial Automation Protocols

## 2.1 Modbus

Modbus RTU / TCP

### Overview

OSI Layer: Layer 7 (Application)

Architecture: Master/Slave (Client/Server)

RTU Transport: RS-485 serial

TCP Transport: Ethernet (port 502)

Speed / Baud Rate: RTU: 9600–115200 bps; TCP: 10/100 Mbps

### Frame Structure (Bit-Level)

| Field | Bytes | Description (RTU) |
| --- | --- | --- |
| Slave Address | 1 | Device ID (1–247) |
| Function Code | 1 | 0x03=Read Holding Regs |
| Data | N | Start addr, quantity, values |
| CRC-16 | 2 | Error check (poly 0xA001) |
| --- | --- | --- TCP (MBAP Header) --- |
| Transaction ID | 2 | Matching ID |
| Protocol ID | 2 | 0x0000 |
| Length | 2 | Remaining bytes |
| Unit ID | 1 | Slave ID |

### C Code Example

/* Modbus CRC-16 */

uint16_t modbus_crc16(const uint8_t *d, uint16_t len) {

uint16_t crc = 0xFFFF;

for (uint16_t i = 0; i < len; i++) {

crc ^= d[i];

for (uint8_t j = 0; j < 8; j++)

crc = (crc & 1) ? (crc>>1)^0xA001 : crc>>1;

}

return crc;

}

int modbus_read_holding(uint8_t *f, uint8_t slave,

uint16_t addr, uint16_t qty) {

f[0]=slave; f[1]=0x03;

f[2]=(addr>>8)&0xFF; f[3]=addr&0xFF;

f[4]=(qty>>8)&0xFF;  f[5]=qty&0xFF;

uint16_t c = modbus_crc16(f, 6);

f[6]=c&0xFF; f[7]=(c>>8)&0xFF;

return 8;

}

### Viva Question

Q: "Coil vs Holding Register?"

A: A Coil is 1-bit R/W (motor ON/OFF). A Holding Register is 16-bit R/W for numerical data (speed setpoint).

## 2.2 PROFINET

### Overview

OSI Layer: Layer 2 (RT/IRT) to Layer 7

Classes: NRT (~100ms), RT (1–10ms), IRT (<1ms)

EtherType: 0x8892

Speed / Baud Rate: 100 Mbps Ethernet; IRT cycle < 1 ms

### Frame Structure (Bit-Level)

| Field | Bytes | Description |
| --- | --- | --- |
| Dest MAC | 6 | Destination |
| Src MAC | 6 | Source |
| EtherType | 2 | 0x8892 |
| Frame ID | 2 | Data relationship ID |
| User Data | N | I/O process data |
| IOPS | 1 | Provider status |
| Cycle Counter | 2 | Cycle count |
| Data Status | 1 | Valid/invalid |
| FCS | 4 | CRC-32 |

### Viva Question

Q: "How does PROFINET IRT achieve sub-ms updates?"

A: Hardware-synchronized clock scheduling splits the cycle into a reserved IRT slot and a standard IT slot.

## 2.3 EtherCAT

Ethernet for Control Automation Technology

### Overview

OSI Layer: Layer 2 (Data Link)

EtherType: 0x88A4

Key Feature: "Processing on the fly" — slaves extract/insert data without buffering

Speed / Baud Rate: 100 Mbps; typical cycle < 100 μs

### Frame Structure (Bit-Level)

| Field | Bytes | Description |
| --- | --- | --- |
| Ethernet Header | 14 | MAC addrs + EtherType |
| EtherCAT Header | 2 | Length + Type |
| Datagram Cmd | 1 | LRW, BRD, FPRD |
| Datagram Address | 4 | Slave addr/offset |
| Datagram Data | N | Process data |
| Datagram WKC | 2 | Working Counter |
| FCS | 4 | CRC-32 |

### Viva Question

Q: "What is 'processing on the fly'?"

A: Dedicated ESC hardware extracts/inserts data while the frame passes through, without buffering the entire frame.

## 2.4 EtherNet/IP

### Overview

OSI Layer: Layer 7 (CIP over TCP/UDP)

Explicit: TCP port 44818

Implicit: UDP port 2222 (real-time I/O)

Speed / Baud Rate: 10/100 Mbps / 1 Gbps

### Viva Question

Q: "Why UDP for I/O instead of TCP?"

A: UDP has lower overhead. Dropping an occasional old frame is better than delaying the stream with TCP retransmissions.

## 2.5 BACnet

Building Automation and Control

### Overview

OSI Layer: Layers 1–7

Transport: MS/TP (RS-485) or BACnet/IP (UDP:47808)

Data Model: Object-Property architecture

Speed / Baud Rate: MS/TP: 9.6–76.8 kbps; IP: 10/100 Mbps

### Viva Question

Q: "What defines a BACnet Object?"

A: A collection of structured Properties (Present_Value, Status_Flags, Object_Type) representing a sensor or function.

# Module 3: IoT & Wireless Protocols

## 3.1 MQTT

Message Queuing Telemetry Transport

### Overview

OSI Layer: Layer 7

Transport: TCP (1883, TLS: 8883)

Architecture: Publish/Subscribe with central Broker

QoS: 0 (at most once), 1 (at least once), 2 (exactly once)

Speed / Baud Rate: Network dependent (TCP)

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| Packet Type | 4 | CONNECT=1, PUBLISH=3, SUBSCRIBE=8 |
| Flags | 4 | DUP(1) + QoS(2) + RETAIN(1) |
| Remaining Length | 8–32 | Variable-length (1–4 bytes) |
| Topic Length | 16 | UTF-8 string length |
| Topic Name | N×8 | e.g. home/temp |
| Packet ID | 16 | QoS > 0 only |
| Payload | M×8 | Application data |

### C Code Example

/* MQTT PUBLISH Builder */

int mqtt_encode_remaining(uint8_t *b, uint32_t l) {

int i=0;

do { uint8_t v=l%128; l/=128;

if(l>0) v|=0x80; b[i++]=v;

} while(l>0); return i;

}

int mqtt_build_publish(uint8_t *buf,

const char *topic, const uint8_t *pay,

uint16_t plen, uint8_t qos, uint16_t pid) {

int o=0;

buf[o++] = 0x30 | ((qos&3)<<1);

uint16_t tl = strlen(topic);

uint32_t rem = 2+tl+plen+(qos>0?2:0);

o += mqtt_encode_remaining(&buf[o], rem);

buf[o++]=(tl>>8)&0xFF; buf[o++]=tl&0xFF;

memcpy(&buf[o], topic, tl); o+=tl;

if(qos>0){buf[o++]=(pid>>8)&0xFF;buf[o++]=pid&0xFF;}

memcpy(&buf[o], pay, plen);

return o+plen;

}

### Viva Question

Q: "Explain the three MQTT QoS levels."

A: QoS 0: at most once (fire-and-forget). QoS 1: at least once (PUBACK, duplicates possible). QoS 2: exactly once (4-step handshake).

## 3.2 CoAP

Constrained Application Protocol

### Overview

OSI Layer: Layer 7

Transport: UDP (5683, DTLS: 5684)

Header: Fixed 4 bytes

Methods: GET, POST, PUT, DELETE

Speed / Baud Rate: Up to 250 kbps (802.15.4)

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| Version | 2 | Always 01 |
| Type | 2 | CON=0, NON=1, ACK=2, RST=3 |
| TKL | 4 | Token length (0–8) |
| Code | 8 | 0.01=GET, 2.05=Content |
| Message ID | 16 | ACK matching |
| Token | 0–64 | Request/response correlation |
| Options | Var | Delta+Length+Value |
| 0xFF + Payload | Var | Payload marker + data |

### Viva Question

Q: "Why CoAP over HTTP for IoT?"

A: CoAP runs over UDP with a 4-byte header, drastically reducing packet size and power consumption vs TCP-based HTTP.

## 3.3 Zigbee

### Overview

Base: IEEE 802.15.4 (LR-WPAN)

Frequency: 2.4 GHz

Modulation: DSSS

Topology: Self-healing Mesh

Roles: Coordinator, Router, End Device

Speed / Baud Rate: 250 kbps

### Viva Question

Q: "How does Zigbee mesh maintain reliability?"

A: Dynamic routing — if a router fails, the network recalculates an alternative multi-hop path.

## 3.4 Z-Wave

### Overview

Frequency: Sub-GHz (908 MHz US, 868 MHz EU)

Topology: Source-routed mesh, up to 232 nodes

Speed / Baud Rate: 9.6 / 40 / 100 kbps

### Frame Structure (Bit-Level)

| Field | Bytes | Description |
| --- | --- | --- |
| Home ID | 4 | Network ID |
| Src Node | 1 | Sender (1–232) |
| Frame Ctrl | 1 | Type/routed/ACK |
| Length | 1 | Remaining bytes |
| Dst Node | 1 | Receiver |
| Cmd Class | 1 | e.g. 0x25=Binary Switch |
| Command | 1 | e.g. 0x01=SET |
| Data | N | Parameters |
| Checksum | 1 | XOR |

### Viva Question

Q: "Sub-GHz advantage over Zigbee 2.4 GHz?"

A: Better wall penetration and zero interference from Wi-Fi/Bluetooth/microwaves.

## 3.5 LoRaWAN

### Overview

Modulation: Chirp Spread Spectrum (CSS)

Range: Up to 15+ km rural

Classes: A (lowest power), B (beacon), C (always on)

Speed / Baud Rate: 0.3 kbps (SF12) – 50 kbps (SF7)

### Frame Structure (Bit-Level)

| Field | Bytes | Description |
| --- | --- | --- |
| MHDR | 1 | MType(3)+RFU(3)+Major(2) |
| DevAddr | 4 | Device address (LE) |
| FCtrl | 1 | ADR, ACK, FOptsLen |
| FCnt | 2 | Frame counter |
| FPort | 1 | 0=MAC, 1–223=App |
| FRMPayload | N | Encrypted data |
| MIC | 4 | AES-CMAC integrity |

### Viva Question

Q: "LoRaWAN trade-off?"

A: Trades bandwidth for range. 15+ km range but only tiny payloads at low data rates.

## 3.6 BLE

Bluetooth Low Energy

### Overview

Frequency: 2.4 GHz ISM (40 channels)

Hopping: Adaptive Frequency Hopping

Data Model: GATT: Profile→Service→Characteristic

Speed / Baud Rate: 1 Mbps (4.x), 2 Mbps (5.x), 125/500 kbps (LE Coded)

### Frame Structure (Bit-Level)

| Field | Bytes | Description (ADV PDU) |
| --- | --- | --- |
| Preamble | 1 | 0xAA |
| Access Address | 4 | 0x8E89BED6 |
| PDU Header | 2 | Type+TxAdd+Length |
| AdvA | 6 | Advertiser MAC |
| AdvData | 0–31 | AD structures |
| CRC | 3 | CRC-24 |

### Viva Question

Q: "How does BLE save power?"

A: Radio is off most of the time — wakes for ultra-short ms transmissions then deep sleeps.

# Module 4: Web & Application Protocols

## 4.1 HTTP/HTTPS

Hypertext Transfer Protocol (Secure)

### Overview

OSI Layer: Layer 7

Transport: TCP (HTTP/1.1, /2), QUIC/UDP (HTTP/3)

Security: TLS encryption

Methods: GET, POST, PUT, DELETE, PATCH

Speed / Baud Rate: Network dependent

### Frame Structure (Bit-Level)

| Field | Bytes | Description (HTTP/2 Frame) |
| --- | --- | --- |
| Length | 3 | Payload size |
| Type | 1 | DATA=0, HEADERS=1 |
| Flags | 1 | END_STREAM, END_HEADERS |
| Stream ID | 4 | (1 reserved + 31-bit) |
| Payload | N | Frame data |

### Viva Question

Q: "TLS handshake?"

A: Verify identities via certificates, agree cipher suites, exchange symmetric session keys (AES-GCM).

## 4.2 WebSocket

### Overview

Transport: TCP (upgraded from HTTP via 101)

Framing: 2–14 byte overhead, full-duplex

Speed / Baud Rate: Network dependent

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| FIN | 1 | Final fragment |
| Opcode | 4 | 0x1=Text, 0x2=Binary, 0x8=Close |
| MASK | 1 | 1=masked (client→server) |
| Payload Len | 7 | 0–125 / 126(2B) / 127(8B) |
| Masking Key | 0/32 | 4-byte XOR key |
| Payload | N×8 | Data |

### Viva Question

Q: "When WebSocket over HTTP?"

A: Real-time bidirectional: live chat, financial tickers, multiplayer games.

## 4.3 gRPC

Google Remote Procedure Call

### Overview

Transport: HTTP/2 (mandatory)

Serialization: Protocol Buffers (binary)

Streaming: Unary, Client, Server, Bidirectional

Speed / Baud Rate: Network dependent (HTTP/2)

### Viva Question

Q: "Why faster than REST?"

A: Protobuf binary serialization (smaller) + HTTP/2 multiplexing + header compression.

## 4.4 GraphQL

### Overview

Transport: HTTP POST to /graphql

Key Concept: Client specifies exact data shape

Speed / Baud Rate: Network dependent

### Viva Question

Q: "Over-fetching vs under-fetching?"

A: Over: unneeded data. Under: multiple calls needed. GraphQL returns exactly requested fields in one request.

# Module 5: IT, Networking & Infrastructure

## 5.1 TCP/IP

Transmission Control Protocol / Internet Protocol

### Overview

OSI Layer: Layer 4 (TCP) + Layer 3 (IP)

Connection: 3-Way Handshake: SYN→SYN-ACK→ACK

Reliability: Seq numbers, ACKs, sliding window, retransmit

Speed / Baud Rate: Physical layer dependent (10 Mbps – 400 Gbps)

### Block Diagram

Client                     Server

│  SYN (seq=x)               │

├───────────────────────────►│

│  SYN-ACK (seq=y,ack=x+1)  │

│◄───────────────────────────┤

│  ACK (ack=y+1)             │

├───────────────────────────►│

│    Connection Established   │

### Frame Structure (Bit-Level)

| Field | Bits | Description |
| --- | --- | --- |
| Src Port | 16 | Sender port |
| Dst Port | 16 | Receiver port |
| Seq Number | 32 | Byte position |
| Ack Number | 32 | Next expected byte |
| Data Offset | 4 | Header len (32-bit words) |
| Flags | 9 | SYN,ACK,PSH,RST,FIN... |
| Window | 16 | Receive buffer size |
| Checksum | 16 | Integrity check |
| Total min | 160 | 20 bytes w/o options |

### Viva Question

Q: "What makes TCP reliable vs UDP?"

A: Sliding window, sequence numbers, mandatory ACKs. Lost packets detected and retransmitted automatically.

## 5.2 DNS

Domain Name System

### Overview

Transport: UDP port 53 (queries), TCP (zone transfers)

Resolution: Recursive or Iterative

Speed / Baud Rate: UDP, < 100 ms typical

### Frame Structure (Bit-Level)

| Field | Bytes | Description |
| --- | --- | --- |
| Transaction ID | 2 | Matching |
| Flags | 2 | QR,Opcode,RD,RA,RCODE |
| QDCOUNT | 2 | Questions |
| ANCOUNT | 2 | Answers |
| QNAME | Var | Domain (label-encoded) |
| QTYPE | 2 | A=1, AAAA=28, MX=15 |
| QCLASS | 2 | IN=1 |

### Viva Question

Q: "Recursive vs Iterative?"

A: Recursive: resolver finds final answer. Iterative: resolver queries step-by-step with referrals.

## 5.3 DHCP

Dynamic Host Configuration Protocol

### Overview

Ports: Server=67, Client=68

Process: DORA: Discover→Offer→Request→Acknowledge

Speed / Baud Rate: UDP over Ethernet/Wi-Fi

### Viva Question

Q: "Why is DHCPREQUEST broadcast?"

A: To notify all other servers the client chose a specific offer, so they release reserved IPs.

## 5.4 BGP

Border Gateway Protocol

### Overview

Transport: TCP port 179

Type: Path-vector routing protocol

Purpose: Inter-AS routing

Speed / Baud Rate: TCP port 179

### Frame Structure (Bit-Level)

| Field | Bytes | Description |
| --- | --- | --- |
| Marker | 16 | All 0xFF |
| Length | 2 | 19–4096 |
| Type | 1 | OPEN=1,UPDATE=2,KEEPALIVE=4 |

### Viva Question

Q: "What is an Autonomous System?"

A: A large network under single administration sharing a unified routing policy.

## 5.5 SNMP

Simple Network Management Protocol

### Overview

Ports: Agent=161, Traps=162

Components: Manager, Agent, MIB

Speed / Baud Rate: UDP 161/162

### Viva Question

Q: "What is an SNMP Trap?"

A: Asynchronous notification from Agent to Manager reporting a critical event without being polled.

# Module 6: File Transfer & Storage Protocols

## 6.1 FTP / SFTP

### Overview

FTP: Control=port 21, Data=dynamic; unencrypted

SFTP: SSH port 22; single encrypted channel

Modes: Active vs Passive

Speed / Baud Rate: Network dependent

### Viva Question

Q: "FTP behind firewalls?"

A: FTP's dynamic data port causes issues — Active Mode's inbound connection gets blocked by firewalls.

## 6.2 NFS

Network File System

### Overview

NFSv3: Stateless, UDP/TCP

NFSv4: Stateful, TCP port 2049, Kerberos

Speed / Baud Rate: TCP port 2049

### Viva Question

Q: "Stateful or stateless?"

A: v2/v3: stateless. v4: stateful for locking and lease management.

## 6.3 SMB

Server Message Block

### Overview

Transport: TCP 445 (direct), 139 (legacy NetBIOS)

Versions: SMB 1.0/CIFS → SMB 3.x (AES encryption)

Speed / Baud Rate: Network dependent

### Frame Structure (Bit-Level)

| Field | Bytes | Description (SMB2 Header) |
| --- | --- | --- |
| Protocol ID | 4 | 0xFE 'S' 'M' 'B' |
| Structure Size | 2 | Always 64 |
| Command | 2 | NEGOTIATE=0, READ=8 |
| Message ID | 8 | Request identifier |
| Session ID | 8 | Auth session |
| Signature | 16 | 128-bit MAC |
| Total | 64 | 512 bits |

### Viva Question

Q: "SMB vs CIFS?"

A: CIFS is an old 1990s dialect of SMB. Modern = SMB v2/v3.

## 6.4 iSCSI

Internet Small Computer Systems Interface

### Overview

Transport: TCP port 3260

Level: Block-level (client mounts as raw disk)

Speed / Baud Rate: 1–10 Gbps (TCP)

### Frame Structure (Bit-Level)

| Field | Bytes | Description (BHS) |
| --- | --- | --- |
| Opcode | 1 | 0x01=SCSI Cmd |
| Flags | 1 | Read/Write/Final |
| Data Seg Length | 3 | Payload |
| LUN | 8 | Logical Unit Number |
| Task Tag | 4 | Command ID |
| CDB | 16 | SCSI Command Block |
| Total BHS | 48 | 384 bits |

### Viva Question

Q: "iSCSI vs NFS?"

A: NFS=file-level (server manages FS). iSCSI=block-level (client manages own FS on raw disk).

# Module 7: Communication & Messaging Protocols

## 7.1 SMTP / IMAP / POP3

Email Protocol Suite

### Overview

SMTP: Send mail (port 25/587)

POP3: Download-and-delete (110/995)

IMAP: Server-side sync (143/993)

Speed / Baud Rate: Network dependent (TCP)

### Viva Question

Q: "IMAP over POP3?"

A: Multi-device sync. IMAP keeps mail on server; changes sync everywhere.

## 7.2 SIP

Session Initiation Protocol

### Overview

Transport: TCP/UDP port 5060; TLS: 5061

Purpose: Signaling only: call setup, teardown

Media: Negotiated via SDP, carried by RTP

Speed / Baud Rate: Network dependent

### Viva Question

Q: "SIP's role if it doesn't carry voice?"

A: SIP is the switchboard: registration, call setup, codec negotiation (SDP), modification (hold), and teardown (BYE).

## 7.3 XMPP

Extensible Messaging and Presence Protocol

### Overview

Transport: TCP 5222 (client), 5269 (server-server)

Format: XML stream: <message>, <presence>, <iq>

Addressing: JID: user@domain.com/resource

Speed / Baud Rate: Network dependent (TCP)

### Viva Question

Q: "Scalable but heavy?"

A: XML streaming is extensible and structured, but verbose tags require more CPU parsing vs binary protocols.

# Appendix: Protocol Speed Quick-Reference

| Protocol | Category | Speed / Baud Rate | Topology |
| --- | --- | --- | --- |
| CAN | Automotive | 1 Mbps (FD:8 Mbps) | Bus |
| LIN | Automotive | 20 kbps | Bus |
| FlexRay | Automotive | 10 Mbps/ch | Dual bus |
| MOST | Automotive | 25–150 Mbps | Ring |
| Auto Eth | Automotive | 100M–10 Gbps | Star |
| Modbus RTU | Industrial | 1.2–115.2 kbps | Bus |
| PROFINET | Industrial | 100 Mbps | Star |
| EtherCAT | Industrial | 100 Mbps | Line/Ring |
| EtherNet/IP | Industrial | 100M–1Gbps | Star |
| BACnet | Building | 9.6–76.8 kbps | Bus/Star |
| MQTT | IoT | TCP dependent | Pub/Sub |
| CoAP | IoT | 250 kbps | C/S |
| Zigbee | IoT | 250 kbps | Mesh |
| Z-Wave | IoT | 9.6–100 kbps | Mesh |
| LoRaWAN | IoT | 0.3–50 kbps | Star-of-stars |
| BLE | IoT | 1–2 Mbps | P2P |
| HTTP | Web | Network dep. | C/S |
| WebSocket | Web | Network dep. | Duplex |
| gRPC | Web | Network dep. | C/S |
| TCP/IP | Network | 10M–400Gbps | E2E |
| DNS | Network | UDP <100ms | Hierarchical |
| BGP | Network | TCP:179 | AS mesh |
| FTP/SFTP | Storage | Network dep. | C/S |
| NFS | Storage | TCP:2049 | C/S |
| SMB | Storage | TCP:445 | C/S |
| iSCSI | Storage | 1–10 Gbps | Init/Target |
| SMTP | Messaging | TCP | Relay |
| SIP | Messaging | TCP/UDP | C/S |
| XMPP | Messaging | TCP | Federated |